Exp. No: 2

Determination and plotting of Electric field and Electric flux density in sphere full of charges with varying radius.

Aim:

To write program for electric field and electric flux density in sphere using

SCILAB.

# Software required:

- SCILAB version 6.0.1

# Program:

clc; clear;

q=input('Enter the value of q='); r=input('Enter the value of r=');

efcilon=8.854*10^-12; pi=3.14

E=q/4*pi*efcilon*r^2; disp("The electric field is", [E]); D=E*efcilon;

disp("The flux density is", [D]);

x = linspace(0, 10, 50);

y = E * exp(-0.1 * x);

z = D * exp(-0.1 * x);

figure(); subplot(2, 2, 1);

plot2d3(x,y);

xlabel("R");

ylabel("Electric field intensity"); title('Electric field intensity'); subplot(2, 2, 2);

plot2d3(x,z);

xlabel("A");

ylabel("Electric flux intensity"); title('Electric flux intensity');

Output:-

Enter the value of q=5*10^-6 Enter the value of r=3

"The electric field is" 4.99D-7

"The flux density is" 4.421D-4

# Calculation:

# Case 1:

Derive and plot the electric field (E) and electric flux density (D) inside a uniformly charged sphere with charge density ρ0. What are the values of E and D at the center and at the surface of the sphere?

# Code:

rho0 = 1e-6; // Uniform charge density (C/m^3) epsilon = 8.854e-12; // Permittivity of free space (F/m) R = 1; // Radius of the sphere (m)

n = 100; // Number of steps for plotting

r = linspace(0, R, n); // Radial distances

// Initialize arrays for D and E

D = zeros(1, n);

E = zeros(1, n);

// Calculate D and E

for i = 1:n

ri = r(i); // Current radius

// Enclosed charge Q_enclosed = rho0 * (4/3) * pi * r^3

Q_enclosed = rho0 * (4/3) * %pi * ri^3;

// Electric Flux Density D

if ri ~= 0 then

D(i) = Q_enclosed / (4 * %pi * ri^2); else

D(i) = 0; // Avoid division by zero end

// Electric Field E = D / epsilon

E(i) = D(i) / epsilon; end

// Plotting

subplot(1, 2, 1);

plot(r, D, "r");

xlabel("Radius (m)");

ylabel("Electric Flux Density (D) [C/m^2]"); title("Uniform Density: D vs. Radius");

subplot(1, 2, 2);

plot(r, E, "b");

xlabel("Radius (m)"); ylabel("Electric Field (E) [V/m]");

title("Uniform Density: E vs. Radius");

# Output:

# Case 2:

For a sphere with a linearly increasing charge density ρv=ρ0r.derive the expressions for the electric field (E) and electric flux density (D). Plot their variations inside the sphere and analyze the trends.

# Code:

// Parameters

rho0 = 1e-6; // Coefficient for linearly increasing charge density (C/m^4)

epsilon = 8.854e-12; // Permittivity of free space (F/m)

R = 1; // Radius of the sphere (m)

n = 100; // Number of steps for plotting

r = linspace(0, R, n); // Radial distances

// Initialize arrays for D and E

D = zeros(1, n);

E = zeros(1, n);

// Calculate D and E

for i = 1:n

ri = r(i); // Current radius

// Enclosed charge Q_enclosed = rho0 * pi * r^4

Q_enclosed = rho0 * %pi * ri^4;

// Electric Flux Density D

if ri ~= 0 then

D(i) = Q_enclosed / (4 * %pi * ri^2); else

D(i) = 0; // Avoid division by zero end

// Electric Field E = D / epsilon

E(i) = D(i) / epsilon;

end

// Plotting

subplot(1, 2, 1);

plot(r, D, "r");

xlabel("Radius (m)");

ylabel("Electric Flux Density (D) [C/m^2]"); title("Linear Density: D vs. Radius");

subplot(1, 2, 2);

plot(r, E, "b");

xlabel("Radius (m)"); ylabel("Electric Field (E) [V/m]"); title("Linear Density: E vs. Radius");

# Output:

# Case 3:

A sphere of radius R=2 m has a uniform charge density ρ=3×10^−6 C/m^3\

- Calculate and plot the electric flux density D(r) and electric field E(r) as a function of radial distance r, ranging from 0 to 4 meters.

- Discuss the behaviour of the electric field inside and outside the sphere.

# Code:

rho0 = 3e-6; // Uniform charge density (C/m^3) epsilon = 8.854e-12; // Permittivity of free space (F/m) R = 2; // Radius of the sphere (m)

n = 200; // Number of steps for plotting

r = linspace(0, 4, n); // Radial distances from 0 to 4 meters

// Initialize arrays for D and E

D = zeros(1, n);

E = zeros(1, n);

// Calculate the total charge enclosed in the sphere

Q_total = rho0 * (4 * %pi * R^3 / 3);

// Calculate D and E

for i = 1:n

ri = r(i); // Current radius

// Enclosed charge for r < R (inside the sphere)

if ri < R then

Q_enclosed = rho0 * (4 * %pi * ri^3 / 3);

// Enclosed charge for r >= R (outside the sphere)

else

Q_enclosed = Q_total; end

// Electric Flux Density D

D(i) = Q_enclosed / (4 * %pi * ri^2);

// Electric Field E = D / epsilon

E(i) = D(i) / epsilon; end

// Plotting

subplot(1, 2, 1);

plot(r, D, "r");

xlabel("Radius (m)");

ylabel("Electric Flux Density (D) [C/m^2]"); title("Uniform Charge Density: D vs. Radius"); subplot(1, 2, 2);

plot(r, E, "b");

xlabel("Radius (m)"); ylabel("Electric Field (E) [V/m]");

title("Uniform Charge Density: E vs. Radius");

# Output:

Result :

Thus the program was verified for electric field and electric flux density in sphere using SCILAB was successfully.